USER GUIDE

SOLUTION 6000

CREATING USERS WITH TOKENS

This User Guide provides the required procedures for administering Users.

1.0 - Creating Users:

  - - Adding a User with a Code

  - - Adding a User with a Token

  - - Checking Token Status

  - - Adding a Door/s to a User

  - - Adding a User’s Name

- - Deleting Users:

  - - Deleting a User Code

  - - Deleting a User Token

  - - Deleting a User Name

3.0 - How To Use a Code:

3.1 - Arming with a Code

  - - Disarming with a Code

- - How To Use a Token:

  - – Arming with a Token

  - - Disarming with a Token

- - How to Exit the System Coding:

1.0 - Creating Users:

Login to the Security System via the Keypad by pressing [Master Code] + [Menu]

1.1 - Adding a User with a Code:

Step 1 - Press [Menu 112] (Add Pin)

Step 2 – Using Up/Down Arrows, select required User. Press [OK]

Step 3 – Enter Code for User. Press [OK]

1.2 - Adding a User with a Token:

Step 1 - Press [Menu 120] (Add Token)

Step 2 – Using Up/Down Arrows, select required User. Press [OK]

Step 3 – Present Token to underside of Keypad.  The Keypad will beep once to indicate the Token has been accepted. Press [OK]

1.3 - Checking Token Status:

To check if Token has been accepted, Press [Menu 122] (Token Status)

1.4 - Assigning Door/s to Users Token:

Step 1 – Press [Menu 145] (Door Assignment)

Step 2 – Using Up/Down Arrows, select required User. Press [OK]

Note:

Salamander Bay has 2 Doors

Step 3 – Using Up/Down Arrows, select required Door (either Door 1 or Door 2). Press [On]. A Tick will be displayed next to the required Door.

Step 4 – Press [OK]

1.5 - Adding a User’s Name

Step 1 – Press [Menu 140] (User Name)

Step 2 - Using Up/Down Arrows, select required User. Press [OK]

Step 3 – Using the Keypad (as per below), enter the User Name.       Press [OK]

2.0 - Deleting Users:

Login to Security System via the Keypad by pressing [Master Code] + [Menu]

2.1 Deleting a User Code:

Step 1 - Press [Menu 113] (Delete Pin)

Step 2 – Using Up/Down Arrows, select required User. Press [OK]

2.2 Deleting a User Token:

Step 1 - Press [Menu 121] (Delete Token)

Step 2 – Using Up/Down Arrows, select required User. Press [OK]

2.3 Deleting a User Name:

Step 1 – Press [Menu 140] (User Name)

Step 2 - Using Up/Down Arrows, select required User. Press [OK]

Step 3 – Using the Keypad (as per below), change the User Name to read “Not Used”.   Press [OK]

3.0 - How to Use a Code:

3.1 - Arming with a Code:

The following procedure may only be completed at the Security System Keypad:

Step 1 – Press your [Code] (four digits)

Step 2 – Press [On]

The Security System will commence the Exit Countdown.

Exit the premises

3.2 - Disarming with a Code:

The following procedure may only be completed at the Security System Keypad after entering the premises:

Step 1 – Press your [Code] (four digits)

Step 2 – Press [Off]

The Security System is now Disarmed

4.0 - How To Use a Token:

  - - Arming with a Token:

Step 1 - At Door Reader, swipe Token 3 Times.   This action will Arm Security System.

Note:  The Door Lock/Electronic Door will open after first swipe.

4.2 - Disarming with a Token:

Step 1 - At Door Reader, swipe Token Once.  This action will Disarm Security System and Activate (Unlock) Door Lock for 15 Seconds.

- - How to EXIT the Security System Coding

Press and hold [Menu]